(in previous chapter describe the set up… The detector of this thesis is a gadolinium-based semiconductor neutron detector. It is designed as a sandwich composed of a slim silicon sensor and a thin gadolinium neutron-converter foil. Neutrons are absorbed in gadolinium and produce electrons and gammas. The reaction products act as neutron indicators by generating a signal in the sensor.)

# Particle Interaction with Matter

In the field of neutron detection, understanding particles interactions with matter is essential. Knowing how and which interactions take place inside a detector makes it possible to predict its response to particles as well as interpreting its measurements of the radiation field.

For a gadolinium-based semiconductor neutron detector, particles of interest are first and foremost neutrons and secondly electrons and photons, reaction products of gadolinium-neutron capture. The following sections give an introduction to neutron, electron and photon interactions with matter, providing a ground understanding of the physics inside the neutron detector in question.

## Neutrons

Neutrons are subatomic particles with zero net charge and a mass comparable to that of a proton. A single neutron is made up of three quarks, one up and two down. The composite quarks accolade to its lack of electric charge, the positive charge of one up quark (+2/3e) cancels the negative charge of two down quarks (2x-1/3e).

The four fundamental forces in physics are: the electromagnetic, the gravitational, the strong and the weak force. Neutrons are mainly affected by the latter two, the nuclear forces. Being neutral, they do not interact electromagnetically, and effects of gravity become insignificant in the shadow of nuclear interactions. The strong and weak force operate at a nuclear level and become effective only when interacting particles are in close proximity of one another; the strong force becomes effective at 10-15cm and the weak force closer still at 10-18cm.  The former is responsible for holding quarks together (to form nucleons) as well as binding protons and neutrons (to form atomic nuclei), while the latter is liable for fusion reactions and atomic-radioactive decay.

Neutron Cross-section

Neutrons reaction probability is heavily dependent on energy and are appropriately grouped into four categorized. Slow neutrons have energies from 0MeV up to 0.4 MeV (Cadmium cut-off region) and their reaction probability drop inversely with their speed. The intermediate neutron energy range is the interval 0.4-200keV. Many reactions have resonances in this interval, and it is therefore also known as the resonance region. Most neutrons, however, are created with a much larger energy (200 keV-20 MeV) and are labeled as fast neutrons. Lastly, beyond 20MeV, they are known as high energy neutrons.

Cross-sections tend to be much higher in the slow and intermediate domain than the fast and high domain. As the neutron speed v increases from slow to intermediate, many reaction probabilities decrease proportionally to 1/v. Once the energy reaches the domain of intermediate neutrons a number of reactions experience resonance. Moving into the fast neutron region, the likelihood of nuclear reactions lessens, and the probability of elastic scattering takes over.

Elastic scattering is widely used in detection of fast neutrons. The interaction produces no secondary particles, but rather results in a recoil nucleus which is responsible for ionizing the detector medium. Fast neutrons have a tendency to scatter more easily of atoms, especially those with a low atomic number such as hydrogen (Ref. on elastic scattering of hydrogen?). + cadmium cutoff?

In the detection of thermal neutrons, reactions like neutron capture are commonly used. Illustrated in figure.. one can see neutron cross-section vs. energy for neutron-gadolinium reactions, capture and elastic scattering.

Neutron capture

In neutron capture an incident neutron is absorbed by a target nucleus. Upon absorption the target nucleus undergoes nuclear compound formation and its nuclear atomic number increases by one atomic unit. The resultant compound nucleus is either stable or unstable. The latter leads to emission of nuclear radiation, heavy ion and/or other fundamental particles. (source? Wiki?)

There are two categories of neutron capture, radiative and nonradiative. Radiative capture reactions produce an excited compound nucleus prone to de-excitation via gamma decay; one or more gamma rays are emitted until the nucleus reaches ground state.

On the other hand, nonradiative capture reactions produce a daughter nucleus accompanied by more than one neutron (since single neutrons emission reactions are considered as inelastic scattering) or charged particles like protons and alphas.

Cross section?

Why it happens?

Some of the significant capture reactions for neutron detection are B-10(n,a)Li*, Li-7(n,a)H-3, He-3(n,p)H.

Neutrons

Neutrons are subatomic particles with zero net charge and a mass comparable to that of a proton. A single neutron is made up of three quarks, one up and two down. The composite quarks accolade to its lack of electric charge, as the positive charge of one up quark (+2/3e) cancels the negative charge of two down quarks (2x-1/3e).

There are plenty of neutron sources, both natural and manmade. Where do they come from….

Particle accelerators

Nuclear weapons

Spallation sources

Complementary to x-rays

including fast neutrons from galactic cosmic ray (GCR)

The four fundamental forces in physics are: the electromagnetic, the gravitational, the strong and the weak force. Neutrons are mainly affected by the latter two, the nuclear forces. Being neutral, they do not interact electromagnetically, and effects of gravity become insignificant in the shadow of nuclear interactions. The strong and weak force operate at a nuclear level and become effective only when interacting particles are in close proximity of one another; the strong force becomes effective at 10-15cm and the weak force closer still at 10-18cm.  The former is responsible for holding quarks together (to form nucleons) as well as binding protons and neutrons (to form atomic nuclei), while the latter is liable for fusion reactions and atomic-radioactive decay.

Neutron interactions

neutrons are neutrally charged. They constitute of two up quarks and one down quark and their collective charge is zero. Without charge, neutrons are oblivious to electromagnetic forces.

- Properties

  - Neutrally charged

  - Indirectly ionizing

  - Deeply penetrating

- Cross-section energy dependence

- Interactions

  - Many interactions

  - Most important in neutron detection are

  - Our detector is based on neutron capture

- Energy dependence

- produce secondary particles (electrons and photons)

Electrons

Photons

A particle traversing matter loses energy and/or is scattered from its original path.

Particles characteristic determine which types of interaction take place and thus also its propagating nature. For instance, particles with electric charge behave quite differently from those without. Charged particles experience intervening Coulomb forces and slow down or stop. On the other hand, neutral particles are indifferent to such forces and because of this, travel much large distances in comparison.

Electron energy loss

Charged particles